# Unit IV

# Timer and Counters:-

- Microcontroller application requires the counting of external events such as frequency of pulse train (clock) or generation of precise internal time delays between computer actions.

- Counting or delay generation can be done by software program / loops, which keeps microprocessor busy and other more important functions are not performed at that time by CPU.

- To relieve this burdon on CPU, 8051 has two 16 – bit timers / counter namely Timer 0 and Timer 1. The timers are used to generate time delay (counts internal clock cycle), whereas counters are used to count clock events that are generated by external device connected to microcontroller.

- Timer 0 / 1 Register: - The 16-bit register of Timer 0 / 1 is accessed as low and high-byte. The low-byte register is called TL0 / TL1 (Timer 0 / 1 low byte) and the high-byte register is called TH0 / TH1 (Timer 0 / 1 high byte). These registers can be accessed like any other register. For example,	MOV TL0, #14H

MOV TH0, #40H MOV TL1, #24H MOV TH1, #10H

Figure 2.6 below shows format of Timer 0 / 1.

| D15 | D14 | D13 | D12 | D11 | D10 | D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH 0 / 1 |  |  |  |  |  |  |  | TL 0 / 1 |  |  |  |  |  |  |  |
| Timer 0 / 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 2.6 Format of Timer 0 / 1.

# TMOD (Timer Mode) register: -

  - It is an 8-bit register used to set timer mode of timer 0 and timer 1.

  - In TMOD the lower 4 bits are set aside for Timer 0 and the upper four bits for Timers.

  - In each case, the lower 2 bits are used to set the timer mode and the upper 2 bits are used to specify operation.

  - Figure 2.7 below shows format of TMOD register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gate 1 | C / T̅ | T1M1 | T1M0 | Gate 0 | C / T̅ | T0M1 | T0M0 |
| Timer 1 |  |  |  | Timer 0 |  |  |  |

Figure 2.7 Format of TMOD register

- Gate (Bit 3, 7):- It is control bit.

If GATE = 1, then it enable Timer 0 / 1 only when the external hardware interrupt INT0

/INT1 pin is high and TR0 / TR1 control bit is set.

0 = Enable Timer/Counter when TR0/TR1 control bit is set.

- C / T̅ (Bit 2, 6): - Timer or Counter select bit. This bit is used to select either timer or counter is in operation.

If (C / T̅) = 1, then counter is selected. If (C / T̅) = 0, then timer is selected.

- M1 & M0 (Bit 4:5 & 0:1): - Timer/Counter mode select bit. These bits are used to select timer mode as shown in table 2.4 below.

| M1 | M0 | Mode | Operation |
| --- | --- | --- | --- |
| 0 | 0 | 0 (13-bit timer mode) | 13-bit timer/counter, 8-bit of THx & 5-bit of TLx |
| 0 | 1 | 1 (16-bit timer mode) | 16-bit timer/counter, THx cascaded with TLx |
| 1 | 0 | 2 (8-bit auto-reload mode) | 8-bit timer/counter (auto-reload mode), TLx reload with the value held by THx each time TLx overflow |
| 1 | 1 | 3 (split timer mode) | Split the 16-bit timer into two 8-bit timers i.e. THx and TLx like two 8-bit timer |

Table 2.4 Timer Mode

- TCON (Timer control) register: - It is 8 – bit register. Figure 2.8 below shows format of TCON register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TF1 | TR1 | TF0 | TR0 | IE1 | IT1 | IE0 | IT0 |

Figure 2.8 Format of TCON register.

- IT0: - External interrupt 0 control bit.

IT0 = 1, it enables external interrupt INT0 to be triggered by falling edge signal. IT0 = 0, it enables low level signal on external interrupt INT0 to generate interrupt.

- IE0: - External Interrupt 0 Edge flag.

IE0 = 1, when a high to low edge signal is received on pin P3.2 (I̅N̅̅T̅̅O̅̅).

IE0 = 0, when processor vectored to ISR (Interrupt Service Routine) located at address 0003H.

- IT1: - External interrupt 1 control bit.

IT1 = 1, it enables external interrupt INT1 to be triggered by falling edge signal. IT1 = 0, it enables low level signal on external interrupt INT1 to generate interrupt.

- IE1: - External Interrupt 1 Edge flag.

IE1 = 1, when a high to low edge signal is received on pin P3.3 (I̅N̅̅T̅̅1̅̅).

IE1 = 0, when processor vectored to ISR (Interrupt Service Routine) located at address 0013H.

- TR0: - Timer 0 run control bit TR0 = 1, it starts timer 0.

TR0 = 0, it halt / stop timer 0 (does not reset timer).

- TF0: - Timer 0 overflow flag.

TF0 = 1, when timer 0 rolls from all ones to all zeros i.e. it overflows its limit.

TF0 = 0, when processor vectors to execute ISR (Interrupt Service Routine) located at address 000BH.

- TR1: - Timer 1 run control bit TR1 = 1, it starts timer 1.

TR1 = 0, it halt / stop timer 01 (does not reset timer).

- TF1: - Timer 1 overflow flag.

TF1 = 1, when timer 1 rolls from all ones to all zeros i.e. it overflows its limit.

TF1 = 0, when processor vectors to execute ISR (Interrupt Service Routine) located at address 001BH.

- Timer Modes: - Timers in 8051 supports four modes of operation as follows: -

- Mode 0 = 13 – bit timer

- Mode 1 = 16 – bit timer

- Mode 2 = 8 – bit auto reload timer

- Mode 3 = 8 – bit split timer.

- Mode 0 = 13 – bit Timer: - It uses 8 – bits of THx and 5 – bits of TLx, here „X‟ denotes timer 0 or 1. This mode holds value between 0000H to 1FFFH, therefore when timer reaches 1FFFH value it rolls over to 0000H and TFx is raised.

- Mode 1 = 16 – bit Timer: - This mode uses 8 – bits of TLx and THx each register, which creates 16 – bit timer. It allows values from 0000H to FFFFH to be loaded in THx and TLx register.

- Mode 2 = 8 – bit Auto Reload Timer: - It allows values from 00H to FFH to be loaded into THx register. When timer reaches FFH value it rools over to 00H and TFx flag is raised, after this timer again loaded with count available in THx register.

- Mode 3 = 8 – bit Split Timer Mode: - In this mode T0 is loaded with delay count and it automatically start T1. T0 and T1 can be programmed in mode 0, 1 and 2 independently, but in mode 3 T0 and T1 do not work independently. T1 in mode 3causes it to stop counting. The control bit TR1 and TF1 overflow flag are then used by timer 0. T0 in mode 3 becomes two completely separate 8 – bit timers. TL0 is controlled by gate arrangement as in figure below and TF0 whenever it rolls over from FFH to 00H.

# Square waveform generation using timer: -

One can generate square wave using timers present in 8051 microcontroller. Square wave can be generated using any mode of timer. we generate square waveform using timer mode 1.

# Timer delay calculation: -

  - 8051 oscillator frequency is 11.0592 MHz, which is divided by 12.

  - time to increment timer count i.e. one tick can be determined as follows: - FT = Fc / 12 = 11.0592MHz / 12

FT = 921 KHz

One tick = 1 / FT = 1 / 921 KHz

# One tick = 1.085µs.

  - Now timer value for required delay can be calculated as

Count = delay / tick Here, one tick = 1.085µs

Register value = Timer Max – Count

Here Register value is count i.e. required to overflow timer. Timer max count if FFFF as it is 16 – bit timer mode.

# So, Register Value = [Timer Max – (delay / 1.085µs)] + 1

In decimal max count is 65535 so for overflow, we use 65536 timer max value i.e.

# Register value = 65536 – (delay / 1.085µs)

This register value in Hexadecimal must be loaded in TLx and THx registers.

- Write assembly language program to generate square waveform of 10 KHz on port 1 with timer 0.

Solution =  Given	Fsq = 10 KHz, Timer to be use is Timer 0

Assume Crystal oscillator frequency Fc = 11.0592 MHz FT = Fc / 12 = 11.059 2MHz / 12

FT = 921 KHz

One Tick = 1 / 921 KHz = 1.085µs

Delay Calculation: -

Given frequency = 10 KHz

T = 1 / 10 KHz = 100µs Here	T = TON + TOFF

As square waveform is to be generated i. e. TON = TOFF So,	TON = 100µs / 2 = 50µs

# Count = 50µs / 1.085µs = 46 Register value = 65536 – 46 = 65490 Register value in Hex = FFD2H

So, to generate square waveform of 10 KHz we have to load TH0 = 0FFH and TL0 = 0D2H

Assembly program to generate square waveform on Port 1: -

ORG 0000H

MOV TMOD, #01H L1: MOV TL0, #0D2H

MOV TH0, #0FFH MOV P1, #00H ACALL DELAY MOV P1, #0FFH ACALL DELAY SJMP L1

DELAY: SETB TR0 WAIT:	JNB TF0, WAIT

CLR TR0 CLR TF0 RET

- Write assembly language program to generate square waveform of 50% duty cycle on pin P2.5 using timer 1.

Solution =	Given, Timer to be use is Timer 0

Assume Crystal oscillator frequency Fc = 11.0592 MHz, Fsq = 2 KHz, FT = Fc / 12 = 11.059 2MHz / 12

FT = 921 KHz

One Tick = 1 / 921 KHz = 1.085µs

Delay Calculation: -

Given frequency = 2 KHz

T = 1 / 2 KHz = 500µs Here	T = TON + TOFF

As waveform to be generated is of 50% duty cycle i. e. TON = TOFF So,	TON = 500µs / 2 = 250µs

# Count = 250µs / 1.085µs = 231 Register value = 65536 – 231 = 65305 Register value in Hex = FF19H

So, to generate square waveform of 2 KHz we have to load TH0 = 0FFH and TL0 = 19H Assembly program to generate square waveform on Port 1: -

ORG 0000H

MOV TMOD, #10H L1: MOV TL1, #19H

MOV TH1, #0FFH CPL P2.5

ACALL DELAY SJMP L1

DELAY: SETB TR1 WAIT:	JNB TF1, WAIT

CLR TR1 CLR TF1 RET

# Interrupt:-

Whenever any device needs services of microcontroller, that device sends a signal to microcontroller; this signal is called as interrupt signal. Upon receiving an interrupt from device, microcontroller completes its current execution and gives services to that device. The program associated with interrupt is called Interrupt Service Routine (ISR) or Interrupt handler.

# Steps for executing an interrupt: -

Upon receiving interrupt, microcontroller executes following steps: -

- Microcontroller finishes the execution of current instruction and saves address of next instruction i.e. contents of PC on stack memory.

- Microcontroller also saves current status of all interrupts internally (not on stack).

- Then microcontroller jumps to fixed location in memory called interrupt vector table, which holds the address of interrupt service routine (ISR).

- Now, microcontroller gets address of ISR from interrupt vector table and microcontroller goes to that location. It starts executing ISR until it reaches last instruction of subroutine, which is RETI (Return from Interrupt).

- After executing RETI instruction, microcontroller returns to address where it was interrupted. First, microcontroller gets back PC contents from stack by popping the top two bytes of stack into PC. Then microcontroller starts to execute from PC address.

# Types of interrupt:-

In 8051 five interrupts are available. But in 8051 datasheet manufacturer includes „Reset‟ as a sixth interrupt.

- Reset: - When this pin is activated, 8051 microcontrollers CPU jumps to address „0000H‟.

# Two timer interrupts TF0 and TF1: -

  - TF0 interrupt is of timer 0 with vector address „000BH‟ and TF1 is of timer 1 with vector address „001BH‟.

# Two Hardware interrupts:-

  - These are external hardware interrupts named INT0 with vector address „0003H‟ and INT1 with vector address „0013H‟ also known as EX1 and EX2 respectively.

  - Pin no. 12 (P3.2) and pin no. 13 (P3.3) in port 3 are used for external hardware interrupts INT0 and INT1 respectively.

# Serial communication interrupt: -

  - It is single interrupt used for both transmit and receive. The address of serial communication interrupt is „0023H‟.

# Interrupt vector table with priorities:-

If two or more interrupts are activated at same time, then which interrupt gets executed first, this is done by using interrupt priority. When 8051 is powered up, the priorities are assigned according to interrupt vector table. We can alter sequence of priority assigned by assigning higher priority to any one of interrupt. Alteration of interrupt priority can be done by programming Interrupt Priority (IP) register. Table 2.5 shows priorities of interrupts as follows:

-

| Priority | Interrupt | Flag | Interrupt vector address | Pin No. |
| --- | --- | --- | --- | --- |
| 1 | Reset | - | 0000H | 9 |
| 2 | INT0 (Ext. int. 0) | IE0 | 0003H | P3.2 (12) |
| 3 | Timer 0 | TF0 | 000BH | ----- |
| 4 | INT1 (Ext. int. 1) | IE1 | 0013H | P3.3 (13) |
| 5 | Timer 1 | TF1 | 001BH | ------ |
| 6 | Serial | TI/RI | 0023H | ------ |

Table 2.5 Interrupt vector table with priorities

In above table limited no. of bytes is assigned for each interrupts. For e.g. total 8-bytes from location 0003H to 000AH is assigned for INT0. Similarly 8-bytes for TF0 is reserved. If ISR is too short to fit in 8-byte then it is placed in vector table, otherwise LJMP instruction is placed in vector table to point address of ISR. Only 3 ROM locations are assigned to reset pin.

- Interrupt Enable (IE) register: - It is 8 – bit register. Programmer can enable or disable interrupts in microcontroller by programming IE register. Figure 2.9 below shows format of IE register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EA | ----- | -(ET2) | ES | ET1 | EX1 | ET0 | EX0 |

Figure 2.9 Format of IE register

- EA (IE.7): - This pin is used to enable or disable interrupts available in 8051 microcontroller. If EA = „0‟, then it disables all interrupt, no interrupt is acknowledged. If EA = „1‟, then each interrupt sources is individually enabled or disabled by setting or clearing its enable bit.

- (IE.6): - Reserved for future use.

- ET2 (IE.5): - This pin is used only in 8052 microcontroller. If ET2 = „0‟, then it disables Timer 2 overflow or interrupt. If ET2 = „1‟, then it disables Timer 2 overflow or interrupt.

- ES (IE.4): - This pin is used to enable or disable serial communication interrupt. If ES = „0‟, then it disables serial communication interrupt. If ES = „1‟, then it enables serial communication interrupt.

- ET1 (IE.3): - It is used to enable or disable Timer 1 overflow / Interrupt. If ET1 = „0‟, then Timer 1 interrupt is disabled. If ET1 = „1‟, then Timer 1 interrupt is enabled.

- EX1 (IE.2): - It is used to enable or disable external hardware interrupt 1. If EX1 = „0‟, it disable external hardware 1 interrupt. If EX1 = „1‟, it enable external hardware 1 interrupt.

- ET0 (IE.1): - It is used to enable or disable Timer 0 overflow / Interrupt. IF ET0 = „0‟, then Timer 0 interrupt is disabled. If ET0 = „1‟, then Timer 0 interrupt is enabled.

- EX0 (IE.0): - It is used to enable or disable external hardware interrupt 0. If EX0 = „0‟, it disable external hardware 0 interrupt. If EX0 = „1‟, it enable external hardware 0 interrupt.

  - Interrupt Enable (IE) register: - It is 8 – bit register. This register is used to alter interrupt priority. If priority bit = 1, then it assigns High priority. If priority bit = 0, then it assigns Low priority. Figure 2.10 below shows format of IP register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ----- | ----- | (PT2) | PS | PT1 | PX1 | PT0 | PX0 |
| IP.7 | IP.6 | IP.5 | IP.4 | IP.3 | IP.2 | IP.1 | IP.0 |

Figure 2.10 Format of IP register

    - IP.7 & IP.6 (	) = These are reserved bits.

    - IP.5 (PT2) = Timer 2 interrupt priority bit. Used only in 8052 microcontroller.

    - IP.4 (PS) = Serial port interrupt priority bit.

    - IP.3 (PT1) = Timer 1 interrupt priority bit.

    - IP.2 (PX1) = External hardware interrupt 1 priority bit.

    - IP.1 (PT0) = Timer 0 interrupt priority bit.

    - IP.0 (PX0) = External hardware interrupt 0 priority bit.

  - Serial Control (SCON) register: - It is 8 – bit register. This register is used to program start bit, stop bit and data bits of data framing. This register also used to set mode in which communication takes place. Figure 2.11 below shows format of SCON register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SM0 | SM1 | SM2 | REN | TB8 | RB8 | TI | RI |

Figure 2.11 Format of SCON register

- SM0 & SM1 bits: - These are D7 and D6 bits of SCON register. These two bits determine framing of data by specifying bits per character, start and stop bits. These bit also used to select mode of serial communication as shown in table 2.6 below: -

| Mode | SM0 | SM1 | Function |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Shift register, baud rate = F / 12 |
| 1 | 0 | 1 | 8 –bit UART, baud rate variable |
| 2 | 1 | 0 | 9 – bit UART, baud rate – F / 32 or F / 64 |
| 3 | 1 | 1 | 9 – bit UART, baud rate variable |

Table 2.6 Serial communication modes

In mode 1, data framing is of 8 –bits, 1 start and 1 stop bit, which makes it compatible with COM port of IBM compatible PC‟s. In mode 1, total 10 bits are transferred 1 start bit, 8 – bit data and 1 stop bit.

- SM2: - It is D5 bit of SCON register. It is multiprocessor communication bit. Set / Clear by program to enable multiprocessor communication in modes 2 and 3. When set to 1 an interrupt is generated if bit 9 of received data is 1, no interrupt is generated if bit 9 is 0. If set to 1 for mode 1, no interrupt will be generated unless valid stop bit is received, clear to 0, if mode 0 is in use.

- REN: - It is receive enable bit. It is D4 bit of SCON register. IF REN = 1, then it receives data. If REN = 0, then it disables reception. This bit can be set by “SETB SCON.4” instruction or cleared by “CLR SCON.4” instruction.

- TB8: - It is D3 bit of SCON register. Transfer bit 8. It is used in serial mode 2 and 3.

- RB8: - It is D2 bit of SCON register. It is not used in mode 0. In mode 1 it is copy of stop bit when 8 – bit data is received. In mode 2 and 3 it is bit 8.

- TI: - It is D1 bit of SCON register. Set to 1 at end of bit 7 in mode 0 and at beginning of stop bit for other modes. Must be cleared by program. When 8051transfer 8 – bit character. It raises TI flag to indicate that it is raised at beginning of stop bit.

- RI: - It is D0 bit of SCON register. When 8051 receives data serially via RxD pin, it gets rid of start and stop bits and places byte in SBUF register. Then it sets RI flag to receive another data. RI raised halfway through stop bit. It is cleared by program.

  - Power Control (PCON) register: - It is 8 – bit register. It contains two power saving modes bits and one serial baud rate control bit. Figure 2. 12 shows format of PCON register.

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMOD | ----- | ----- | ----- | GF1 | GF0 | PD | IDL |

Figure 2.12 Format of PCON register.

- Bit 7 (SMOD): - It is used to select baud rate. If SMOD = 1, then it doubles baud rate i.e. in this condition 1/12 crystal frequency is divided by 16 and that frequency is used by T1 to set baud rate. If SMOD = 0, then normal baud rate i.e. 1/12 crystal frequency is divided by 32 and that frequency is used by T1 to set baud rate. Table 2.7 shows baud rate selection table

| TH1 (Decimal) | TH1 (Hex) | SMOD = 0 | SMOD = 1 |
| --- | --- | --- | --- |
| -3 | FD | 9600 | 19200 |
| -6 | FA | 4800 | 9600 |
| -12 | F4 | 2400 | 4800 |
| -24 | E8 | 1200 | 2400 |

Table 2.7 Baud rate selection table

- Bit 4 to 6: - Not used / implemented.

- Bit 2 & 3 (GF0 & GF1): - It is general purpose flag bits. It is set by program.

- Bit 1: - It is power down mode bit, if PD = 1, then power down mode is enabled. If PD = 0, then power down mode is disabled.

- Bit 0: - It is Idle mode bit, if IDL = 1, then idle mode is activated. If IDL = 0, then idle mode is disabled.

| PD | IDL | Status |
| --- | --- | --- |
| 0 | 0 | Normal Power Mode |
| 0 | 1 | Idle Mode |
| 1 | 0 | Power Down Mode |
| 1 | 1 | Power Down Mode |